MATERIAL SAFETY DATA SHEET

SECTION I:	CHEMICAL PRODUCT AND MANUFACTURING INDENTIFICATION

PRODUCT NAME:		GEAR OIL 100

PRODUCT CLASS:		MACHINE LUBRICANT

MSDS NO.:			MS263

MANUFACTURER’S NAME:	DOALL COMPANY

ADDRESS:			1480 S. WOLF ROAD

WHEELING, IL  60090

EMERGENCY PHONE:		(800) 535-5053 (INFOTRAC)

GENERAL INFORMATION:	(800) 923-6255

REVISION DATE:		JUNE 2010

SECTION II:	COMPOSITION / INFORMATION ON INGREDIENTS

| INGREDIENTS | CAS # | OSHA PEL | ACGIH TLV | PERCENT |
| --- | --- | --- | --- | --- |
| SOLVENT REFINED PARAFFINIC OIL | 64742-57-0 | 5 MG/M3 | 5 MG/M3 | 85-95 |
| ZINC DIALKYL DITHIOPHOSPHATE | 68457-79-4 | ---- | ---- | 1-3 |
| BARIUM SUFONATE | 61790-48-5 | ---- | ---- | 1-3 |

SECTION III:	HAZARD IDENTIFICATION

HMIS RATING:		2   -   1   -   1

2

NFPA RATING:		2   -   1   -   1

1

1

PRIMARY ROUTE(S) OF ENTRY:  SKIN CONTACT, SKIN ABSORPTION, 					INHALATION, INGESTION, EYE CONTACT.

ACCUTE TOXICITY:  NOT DETERMINED

EFFECTS OF OVEREXPOSURE:

EYE CONTACT:	CONTACT MAY CAUSE MINOR IRRITATION. SEEK MEDICAL ATTENTION IF PROBLEM PERSISTS.

SKIN CONTACT:		PROLONGED OR REPEATED CONTACT CAN RESULT IN DEFATTING AND 					DRYING OF THE SKIN WHICH MAY RESULT IN SKIN IRRITATION AND 					DERMATITIS (RASH).  IF AN EVENT OCCURS SEEK MEDICAL ATTENTION.

INHALATION:			PROLONGED INHALATION MAY CAUSE BREATHING DIFFICULTY.

INGESTION:	MAY CAUSE NAUSEA AND/OR DIARRHEA.  SEEK MEDICAL ATTENTION IF PROBLEM PERSISTS.

SECTION IV:	FIRST AID INFORMATION

INHALATION:		REMOVE INDIVIDUAL TO FRESH AIR.  GIVE OXYGEN IF BREATHING IS LABORED.  			IF BREATHING HAS STOPPED, GIVE ARTIFICAL RESPIRATION.  DO NOT USED MOUTH-			TO-MOUTH METHOD IF VICTIM INHALED THE SUBSTANCE.   KEEP PERSON WARM, 			QUIET AND SEEK MEDICAL ATTENTION.

EYES:			REMOVE CONTACT LENSES.  FLUSH WITH LARGE AMOUNTS OF WATER FOR AT LEAST 			20 MINUTES.  LIFTING UPPER AND LOWER LIDS OCCASIONALLY.  SEEK MEDICAL 			ATTENTION.

SKIN:			THOROUGHLY WASH EXPOSED SKIN WITH SOAP AND WATER.  SEEK MEDICAL 				ATTENTION.

IF SWALLOWED:	IF MATERIAL IS INGESTED, IMMEDIATELY CONTACT A POISON CONTROL CENTER.  			RINSE MOUTH.  DO NOT INDUCE VOMITTING WITHOUT THE ADVICE FROM POISON 			CONTROL CENTER.  IF VOMITING OCCURS NATURALLY, HAVE VICTIM LEAN FORWARD 			TO REDUCE RISK OF ASPIRATION.  DO NOT USE MOUTH-TO-MOUTH METHOD IF VICTIM 			INGESTED THE SUBSTANCE. INDUCE ARTIFICAL RESPIRATION WITH THE AID OF A 			POCKET MASK EQUIPPED WITH A ONE-WAY VALVE OR OTHER PROPER RESPIRATORY 			MEDICAL DEVICE.  NEVER GIVE ANYTHING TO AN UNCONSCIOUS PERSON.  SEEK 			MEDICAL ATTENTION.

SECTION V:	FIRE FIGHTING INFORMATION

FLASH POINT:			475oF (COC) Estimated

FLAMMABLE LIMITS:		NOT ESTABLISHED

AUTOIGNITION TEMP: 	NOT DETERMINED.

EXTINGUISHING MEDIA:	CO2, DRY CHEMICAL, OR FOAM.

FIREFIGHTING:	IN THE EVENT OF FIRE AND/OR EXPLOSION DO NOT BREATHE FUMES.  WEAR 				FULL PROTECTIVE CLOTHING, INCLUDING HELMET, SELF-CONTAINED POSITIVE 				PRESSURE OR PRESSURE DEMAND BREATHING APPARATUS, PROTECTIVE 				CLOTHING AND FACE MASK.  CONTAINERS SHOULD BE COOLED WITH WATER. 				FOR MASSIVE FIRE, USE UNMANNED HOSE HOLDERS OR MONITOR NOZZLES;  				COOL CONTAINERS WITH FLOODING QUANTITIES OF WATER UNTIL WELL 				AFTER FIRE IS OUT.

HAZARDOUS COMBUSTIBLE PRODUCTS:	OXIDES OF CARBON AND SULFUR COMPOUNDS.

SECTION VI:	SPILL OR LEAK PROCEDURES

GENERAL SPILL PROCEDURE:	ISOLATE SPILL OR LEAK AREA IMMEDIATELY.  KEEP UNAUTHORIZED 					PERSONNEL AWAY.  VENTILATE AREA AND ELIMINATE ALL IGNITION 					SOURCES.  STOP LEAK IF YOU CAN DO SO WITHOUT RISK.  MOVE THE 					CYLINDER TO A SAFE AND OPEN AREA IF THE LEAK IS IRREPARABLE.  					STOP THE FLOW OF MATERIAL, IF THIS IS WITHOUT RISK.  USE WATER 					SPRAY TO REDUCE VAPORS OR DIVERT VAPOR CLOUD DRIFT.  						PREVENT ENTRY INTO WATERWAYS, SEWERS, BASEMENTS OR 						CONFINED AREAS.

SMALL SPILLS:			WIPE UP WITH ABSORBENT MATERIAL (E.G. CLOTH, FLEECE).  CLEAN 					CONTAMINATED SURFACE THOROUGHLY.  AFTER REMOVAL FLUSH 					CONTAMINATED AREA THOROUGHLY WITH WATER.

LARGE SPILLS:			SHOULD NOT BE RELEASED INTO THE ENVIRONMENT.  DIKE AND PUMP 					INTO DRUMS.

SECTION VII:	HANDLING AND STORAGE

KEEP CONTAINER CLOSED.  STORE IN A COOL, WELL VENTILATED PLACE. KEEP AWAY FROM HEAT, SPARKS, AND FLAME.  EMPTY CONTAINERS MAY CONTAIN RESIDUE.

SECTION VIII:	EXPOSURE CONTROLS

| OCCUPATINAL EXPOSURE LIMITS |  |  |  |  |
| --- | --- | --- | --- | --- |
| COMPONENTS | CAS # | OSHA | ACGIH | OTHER |
| SOLVENT REFINED PARAFFINIC OIL | 64742-57-0 | 5 MG/M3 | 5 MG/M3 |  |

PROVIDE GENERAL AND/OR LOCAL EXHAUST VENTILATION TO CONTROL AIRBORNE LEVELS BELOW THE EXPOSURE GUIDELINE.

RECOMMENDED FOR PROLONGED OR REPEATED CONTACT.  THE USE OF NEOPRENE GLOVES THAT ARE IMPERMEABLE TO OIL IS RECOMMENDED OR BARRIER CREAM.  SAFETY GOGGLES OR FACE SHIELD ARE RECOMMENDED.  AN EMERGENCY SHOWER AND EYEWASH SHOULD BE AVAILABLE IN THE WORK AREA.  PRACTICE REASONABLE PERSONAL HYGIENE.  WASH EXPOSED SKIN WITH MILD SOAP AND WARM WATER ONLY.

CONTAMINATED EQUIPMENT/GEAR:  LAUNDER ALL CONTAMINATED CLOTHING PRIOR TO RE-USE.

SECTION IX:	PHYSICAL AND CHEMICAL PROPERTIES

APPEARANCE:			AMBER LIQUID

ODOR:				PETROLEUM

PHYSICAL STATE:		LIQUID

VAPOR PRESSURE:		NONE DETERMINED

SPECIFIC GRAVITY:		0.889 (WATER = 1)

VAPOR DENSITY:		>1 (AIR = 1)

EVAPORATION RATE:		NONE DETERMINED (BuAc=1)

SOLUBILITY:			INSOLUBLE

pH:				NOT APPLICABLE

MELTING POINT:		NOT APPLICABLE

BOILING POINT:		NONE DETERMINED

SECTION X:	STABILITY AND REACTIVITY

STABLE UNDER NORMAL CONDITIONS.  KEEP AWAY FROM HEAT, SPARKS OR OPEN FLAMES.

AVOID CONTACT WITH STRONG OXIDIZING AGENTS

HAZARDOUS DECOMPOSITION PRODUCTS:	 OXIDES OF CARBON AND SULFUR COMPOUNDS.

HAZARDOUS POLYMERIZATION WILL NOT OCCUR.

SECTION XI:	TOXICOLOGY

CARCINOGENICITY:		LISTED BY NTP/IARC/OSHA:  NO

SECTION XII:	ECOLOGICAL

NOT DETERMINED.

SECTION XIII:	DISPOSAL

TREAT AS AN OILY WASTE.  EMPTY CONTAINERS MAY BE DISPOSED OF THROUGH NORMAL CHANNELS.  FULL OR PARTICALLY FULL CONTAINERS ARE CONSIDERED HAZARDOUS WASTE AND MUST BE DISPOSED OF IN ACCORDANCE WITH LOCAL, STATE, AND FEDERAL REGULATIONS.

SECTION XIV:	TRANSPORTATION

DEPARTMENT OF TRANSPORTATION:  NOT REGULATED.

SECTION XV:	REGULATORY

CAA:		CONTAINS NO OZONE DEPLETING SUBSTANCE

DOT:		NOT CLASSIFIED AS HAZARDOUS

RCRA:		AS RECEIVED, NOT A HAZARDOUS WASTE MATERIAL

SARA:		CONTAINS 2% BARIUM SULFONATE (61790-48-5)

CONTAINS 2% ZINC DIALKYL DITHIOPHOPHATE (68457-79-4)

TSCA:		ALL INGREDIENTS ARE REGISTERED

SECTION XVI:	OTHER INFORMATION

THIS DOCUMENT HAS UNDERGONE SIGNIFICANT CHANGES AND SHOULD BE REVIEWED IN ITS ENTIRETY.

DATA IN THIS MATERIAL SAFETY DATA SHEET IS BELIEVED TO BE CORRECT AND RELIABLE.  HOWEVER, THE DOALL COMPANY DOES NOT ASSUME RESPONSIBILITY FOR IT, OR ANY RECOMMENDATIONS CONTAINED IN IT, INASMUCH AS CONDITIONS AND METHODS OF USE ARE BYOND OUR CONTROL.  FURTHER, WE MAKE NO WARRANTY, EXPRESSED OR IMPLIED, OF ANY KIND REGARDING THIS PRODUCT OR ITS USE, AND PURCHASER ASSUMES ALL RISKS OF USE OR HANDLING EITHER IN ACCORDANCE WITH DIRECTIONS OR NOT.  THE DATA PROVIDED ON THIS MATERIAL SAFETY DATA SHEET IS NOT MEANT TO BE USED AS SPECIFICATIONS, ONLY AS GUIDELINE INFORMATION AS TO THE SAFE USE OF THE PRODUCT.  THE USER SHOULD REFER TO APPLICABLE  LAWS BEFORE USE.  THIS MATERIAL SAFETY DATA SHEET COMPLIES WITH GUIDELINES SET BY 29CFR1910.1200.